COMM8.DRV

8-Port COM-Driver for Windows 3.1

The Windows operating system currently supports only 4 COM-Ports while the Windows API is ready to support COM1 to COM9. This is because the device driver COMM.DRV only supports 4 ports. When you substitute COMM.DRV with our COMM8.DRV it’s possible to use 8 ports in the same way (using the same API function) you use 4 ports today.

Attention: Unfortionatly many Windows programs today do not check wether or not there are more than four ports available. Although checking is possible it is normally avoided because the standard COMM.DRV does not support more than four ports. Theese programs (including windows accessory TERMINAL.EXE) will not be able to use more than four ports even with COMM8.DRV installed. When writing your own programs you don’t have this problem.

Windows for Workgroups 3.11

Although the COMM8.DRV is based on the Windows 3.1 COMM.DRV it will still work with Windows for Workgroups 3.11.

Installation

To install the COMM8.DRV copy COMM8.* (*= .DRV, *.EXE, *.CPL) from the distribution disk to the WINDOWS\SYSTEM directory and modify the line commdrv=comm.drv  in section [BOOT] of SYTEM.INI to commdrv=comm8.drv .

You can then start the system control panel and select enhanced ports configuration. Then you can choose the additional I/O Base addresses and irq’s for COM1..COM8. The new enhanced ports configuration control panel application will change the [386-Enh]-section of SYTEM.INI to inform windows about your configuration of COM-Port adresses and IRQ’s since there are no standard adresses for COM4..COM8. See the next section for a short guide to SYSTEM.INI COM-Port values.

COM-Port related Entrys in SYSTEM.INI, section [386enh]

COM1Base=address

..

COM8Base=address

These entries specify the base (first) port for the serial port adapter you are using for both standard mode and 386 enhanced mode. Check your hardware documentation for the appropriate value. The defaults are COM3Base=3E8h, and the port address values in the BIOS data area for COM1, COM2, and COM4. To change this entry, choose the Ports icon in Control Panel, and click the Advanced button for the selected COM port.

COM1FIFO=Boolean

..

COM8FIFO=Boolean

These entries specify whether the FIFO buffer of a COM portÆs 16550 Universal Asynchronous Receiver Transmitter (UART) should be enabled (On) or disabled (Off). If a serial port does not have a 16550 UART, this setting is ignored. These values are used by Windows for both standard and enhanced modes. The default is On.

COM1Irq=number

..

COM8Irq=number

These entries specify which interrupt line is used by the device on the specified serial port. Check your hardware documentation for the correct value. If there is a hardware conflict between ports, set a value of _1 to disable input for that COM port. The defaults for ISA and EISA machines are COM1Irq=4, COM2Irq=3, COM3Irq=4, and COM4Irq=3; for MCA machines, the defaults are COM1Irq=4, COM2Irq=3, COM3Irq=3, and COM4Irq=3. To change this entry, choose the enhanced ports configuration icon in Control Panel.

COMIrqSharing=Boolean

This entry specifies whether COM interrupt lines will be sharable between mulitiple serial ports or with other devices. Set this switch if your machine uses the same interrupt for COM3 or COM4 as it does for COM1 or COM2. The default is On for Micro Channel and EISA machines; Off for all other machines. In the enhanced ports configuration will change this entry automatically to TRUE when you select the same IRQ for more than one port.

Sample section of SYSTEM.INI for use with 2 * 4COM

A possible configuration could be like this:

MaxComPort=8

Com3Base=3E8

Com3Irq=7

Com3FIFO=1

Com4Base=2E8

Com4Irq=5

Com4FIFO=1

Com5Base=2F0

Com5Irq=10

Com5FIFO=1

Com6Base=3E0

Com6Irq=11

Com6FIFO=1

Com7Base=2E0

Com7Irq=12

Com7FIFO=1

Com8Base=260

Com8Irq=15

Com8FIFO=1

It is mandatory that theese values are corretly set. Incorrect values can cause system crashes.  COM1 and COM2 are not set, so default values are assumed from windows (COM1, 3F8h, IRQ4 and COM2, 2F8h, IRQ3).

IRQ-sharing

It is possible to set more than one port to the same IRQ by setting the COMxIrq values accordingly and setting ComIRQsharing=True

in the SYSTEM.INI.